
NTABANKULU LOCAL MUNICIPALITY
BID NO: NLM/DP/CFMPS/2026/2027
CONSTRUCTION OF FENCED MATHOLE PRE-SCHOOL WITH ABLUTION FACILITIES
Part C2: Pricing Data
	Content

	
	

	C2.1


	Pricing Instructions
	

	C2.2
	Bill of Quantities
	


NTABANKULU LOCAL MUNICIPALITY
BID NO: NLM/DP/CFMPS/2026/2027
CONSTRUCTION OF FENCED MATHOLE PRE-SCHOOL WITH ABLUTION FACILITIES
C2.1 Pricing Instructions

1. For the purposes of this schedule of quantities, the following words shall have the meanings hereby assigned to them:

Unit: 
The unit of measurement for each item of works as defined in the specifications.

Quantity: 
The number of units of work for each item.

Rate: 
The payment per unit of measurement at which the tenderer tenders to do the work.

Amount: 
The product of the quantity and the rate tendered for an item.

Lump sum: 
An amount tendered for an item, the extent of which is described in the schedule of quantities, the specifications or elsewhere but the quantity of work of which is not measured in any units.

2. This schedule of quantities forms an integral part of the contract documents.

3. The quantities set out in the schedule of quantities are approximate quantities only. The quantities of work finally accepted and certified for payment, and not the quantities given in the schedule of quantities, shall be used for determining payment to the contractor.

The validity of the contract shall in no way be affected by differences between the quantities in the schedule of quantities and the quantities finally certified for payment. Works shall be valued at the rates or lump sums tendered, subject only to the provisions for the general conditions of contract, paragraphs 11 and 12 of this preamble, and sub clause 1209(a) of the standard specifications.

4. Rates and lump sums shall be comprehensive in accordance with sub clause 1209(b) of the standard specifications. Full compensation for completing and maintaining, during the maintenance period, all the work shown in the specifications, and for all the risks, obligations and responsibilities specified in the general conditions of contract, special conditions of contract and specifications shall be considered as provided for collectively in the items of payment given 
5. in the schedule of quantities, except in so far as the quantities given in the schedule of quantities are only approximate.

6. The tenderer shall fill in a rate or a lump sum for each item where provision has been made for it, even where no quantities are given.

A tender may be rejected if the tenderer has not filled in a rate or a lump sum against each item and if the tenderer fails to furnish the rates or lump sums for such items within seven (7) days of having been notified in writing by the employer to do so.

The tenderer shall fill in a rate against all items where the words "rate only" appears in the amount column.

The provisions of sub clause 1209(f) of the standard specifications shall apply to rate-only items.

Tenderers shall also note the provisions of paragraph 12 of this preamble.

The tenderer may not group a number of items together and tender one lump sum for such group of items.

The tenderer also may not indicate against any item that full compensation for such item has been included in another item.

The tendered lump sums and rate shall be valid irrespective of any change in the quantities during the execution of the contract.

7. The works as executed will be measured for payment in accordance with the methods described in the contract documents under the various payment items, notwithstanding any custom to the contrary. Attention is directed to the provisions of clause 1220 of the standard specifications regarding the measurement of quantities for payment. Except where otherwise specified as in clause 1220, the nett measurement or mass of the finished work in place shall be taken for payment but excluding any volume or mass of work in excess of that ordered.

8. The amount of work or the quantities of material stated in the schedule of quantities shall not be considered as restricting or extending the amount of work to be done or quantity of material to be supplied by the contractor.

9. The stating of quantities of material or amount of work in the schedule of quantities shall not be regarded as authorization for the contractor to order material or to execute work. The contractor shall obtain the Employer's detailed instructions for all work before ordering any materials for or executing work or making arrangements in this regard.

10. The short descriptions of the payment items given in the schedule of quantities are only for the purposes of identifying the items and providing specific details. Reference shall be made, inter alia, to the standard specifications, project specifications, general conditions of contract and 
special conditions of contract for more detailed information regarding the extent of the work entailed under each item.

11. Reference shall be made to clause 48 of the general conditions of contract regarding provisional sums and prime cost sums.

12. Subject to the conditions state below, the rates and lump sums filled in by the tenderer in the schedule of quantities shall be final and binding and may not be adjusted should there be any mistakes in the extensions thereof. Should there be any discrepancies between the correctly extended totalled schedule of quantities and the tender sum, the rates will be regarded as being correct, and the employer shall have the right to make such adjustments to the tender sum as he may deem necessary in order to reconcile the total of the schedule of quantities with the tender sum. In such an event the contractor will be consulted but failing agreement between the parties, the decision of the employer shall be final and binding. Such adjustment of the tender sum shall take place only after acceptance of the tender, but prior to the signing of the contract. In their own interest tenderers should make double sure of the correctness of their tendered rates, the extensions and the tender sum.

13. A tender may be rejected if the unit rates or lump sums for some of the items in the schedule of quantities are, in the opinion of the employer, unreasonable or out of proportion, and the tenderer fails, within a period of seven (7) days of having been notified in writing by the employer to adjust the unit rates or lump sums for such items, to make such adjustments.

14. The units of measurement indicated in the schedule of quantities are metric units. The following abbreviations are used in the schedule of quantities:

mm 
= 
millimetre 

h
 = 
hour

m 

= 
metre 

kg
 = 
kilogram

km 

= 
kilometre 

t 
= 
ton (1000 kg)

m² 

= 
square metre 
No.
 = 
number

m².pass 
= 
square metre-pass 
sum 
= 
lump sum

ha

 = 
hectare 

MN 
= 
meganewton

m³ 

= 
cubic metre 
MN.m 
= 
meganewton-metre

m³.km 
= 
cubic metre-kilometre 
P C sum 
= 
Prime Cost sum

l

 = 
litre 

Prov sum 
= 
Provisional sum

kl

 = 
kilolitre 

% 
= 
per cent

MPa
 = 
megapascal 
kW 
= 
kilowatt

15. All rates and sums of money quoted in the schedule of quantities shall be in rands and whole cents. Fractions of a cent shall be discarded.

NB: TENDERERS MUST COMPLETE THE SCHEDULE OF QUANTITIES IN BLACK INK
NTABANKULU LOCAL MUNICIPALITY
BID NO: NLM/DP/CFMPS/2026/2027
CONSTRUCTION OF FENCED MATHOLE PRE-SCHOOL WITH ABLUTION FACILITIES
C2.2 Bill of Quantities

	CONTENTS


	

	BILL OF QUANTITIES
	

	
	

	
	


NTABANKULU LOCAL MUNICIPALITY
BID NO: NLM/DP/CFMPS/2026/2027
1. BILL OF QUANTITIES FOR CONSTRUCTION OF NEW PRE SCHOOL AND RELATED WORKS
	
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	
	
	
	
	
	

	
	
	
	
	
	

	Item No
	Description
	Unit
	Quantity
	Rate
	Amount

	 
	SECTION 1300:
	 
	 
	 
	 

	
	
	
	
	 
	 

	
	
	
	
	 
	 

	13.01
	Security Officer
	Month
	3
	 4000.00
	 

	
	 
	 
	 
	 
	 

	CLO
	Community Liaison Officer
	Month
	3
	 5000.00
	 

	 
	 
	 
	 
	 
	 

	PSC
	PSC attending the monthly meetings
	Month
	3
	 
	 

	 
	 
	 
	 
	 
	 

	OHS
	OHS Plan and its implementation
	Month
	3
	 
	 

	 
	 
	 
	 
	 
	 

	 
	SECTION 1400:
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	14.01 

	Provisioning of site offices with tables and chairs to be utilised by the employer’s representative and the contractor with a first aid box, Storage Sheds, Ablution Facilities, Tools and Equipment, Water, Electricity Power and Communication, Access, Plant
DE-ESTABLISHMENT: Removal of site establishment on completion

	Month
	3
	 
	 

	
	
	
	
	
	 


a. FOUNDATIONS
	ITEM
	DESCRIPTION 
	UNITS
	 QTY 
	RATE
	 TOTAL

	1.
	Clear site
	M2
	233
	
	

	2.
	Excavate for trenches n.e 2m deep
	M3
	27
	
	

	3.
	Concrete footings 20 mPa
	M3
	23
	
	

	4.
	Backfilling to trenches
	M3
	13
	
	

	5.
	Hardcore
	M3
	19
	
	

	6.
	50 mm thick sand blinding
	M2
	103
	
	

	7.
	220mm thick brick wall
	M2
	76
	
	

	8.
	Galvanised brick force every 2nd course
	M
	186
	
	

	9.
	Concrete Surface bed 25mPa
	M3
	11
	
	

	10.
	250 micron DPM
	M2
	105
	
	

	11.
	25 mm thick screed
	M2
	103
	
	

	12.
	375 micron DPC to receive 150 mm block wall
	M
	75
	
	

	13.
	Keep excavation free from water
	Item
	1
	
	

	14.
	Testing of concrete cubes
	No
	6
	
	

	TOTAL
	
	
	
	
	

	
	Bill carried to next summary
	
	
	
	

	b. SUPERSTRUCTURE

	1.
	150 mm block wall
	M2
	181.4
	
	

	2.
	Aluminium window with glazing and fittings size 1.511m x 1.245m
	No
	4
	
	

	3.
	Aluminium window with glazing and fittings size 1.511m x 0.949m
	No
	1
	
	

	4.
	Aluminium window with glazing and fittings size 1.511m x 0.654m
	No
	4
	
	

	5.
	Hard wood Meranti door frame
	No
	6
	
	

	6.
	Meranti doors
	No
	6
	
	

	7.
	3 lever mortice door locks(SABS Approved)
	No
	6
	
	

	8.
	Precast concrete lintels n.e 2m long
	No
	8
	
	

	9.
	Precast concrete lintels n.e 1.5m long
	No
	6
	
	

	10.
	Ceramic floor tiles
	M2
	180
	
	

	11.
	Brass hinges
	Pairs
	6
	
	

	12.
	32mm Rubber door stopper
	No
	6
	
	

	13.
	Plaster walls internal and external
	M2
	296
	
	

	14.
	Plaster to reveals 
	m
	78.4
	
	

	15.
	Galvanised Brick force to every 3 courses
	m
	590
	
	

	16.
	Supply fully fitted distribution box, Tubing, wiring and connection of electricity to Eskom line
	Item
	
	
	

	17.
	Casting of concrete apron around the structure
	m3
	32
	
	

	18.
	Weather guards for external doors
	No
	2
	
	

	19.
	Steel burglar proofing for external doors
	No
	2
	
	

	20.
	Installation of granite handover plaque written in white wording with municipal logo engraved on the wall with rail and sliding curtain
	No
	1
	
	

	Total
	
	
	
	
	

	
	Bill carried to next summary
	
	
	
	

	c. ROOFING

	1.
	Treated S.A  pine  wall plate 114 mm x 38 mm
	m
	31
	
	

	2.
	Truss anchors double strand of 4mm galvanised wire ties
	No
	48
	
	

	3.
	Treated Gang nail roof truss with 22.50pitch at 690mm cc
	No
	24
	
	

	4. 
	Treated Bracing 114X38mm
	m
	30
	
	

	5.
	50x76mm SA pine treated timber purlins 
	m
	246
	
	

	6.
	Ridge capping
	m
	16
	
	

	7.
	0.5mm thick corrugated Roof sheeting
	M2
	216
	
	

	8.
	PVC Fascia board
	m
	31
	
	

	9.
	PVC Bargeboard L-Shape
	m
	28
	
	

	Total
	
	
	
	
	

	
	Bill carried to next summary
	
	
	
	

	d. CEILING

	1.
	38 X 38mm treated SA pine branderings/ battons at 400 mm cc
	M2
	128
	
	

	2.
	4mm Cladit sheets
	M2
	128
	
	

	3.
	75mm gypsum cover cornice
	m
	106
	
	

	4.
	Pine cover strips
	m
	96
	
	

	5.
	600 x 600mm door trap with complete frame
	No
	1
	
	

	6.
	Meranti skirting with quadrant
	m
	106
	
	

	Total
	
	
	
	
	

	
	Bill carried to next summary
	
	
	
	

	e. PAINTING

	1.
	3 coats ceiling paint (white)
	M2
	117
	
	

	2.
	2 coats undercoat to internal walls
	M2
	196
	
	

	3.
	2 coats high quality matt finishing coats to internal walls
	M2
	196
	
	

	4.
	Painting to doors and door frames with 3 coats high gloss enamel paint
	m
	10. 18
	
	

	5.
	2 coats undercoat to external walls
	M2
	98
	
	

	6.
	2 coats high quality matt finishing coats to external walls
	M2
	98
	
	

	7.
	3 coats high gloss enamel paint to doors
	M2
	24
	
	

	8.
	Paint to meranti skirting and quadrant
	m
	106
	
	

	9.
	Painting of roof sheets with galvanized iron primer
	M2
	216
	
	

	10.
	Painting of roof sheets with rubber roof paint
	M2
	216
	
	

	Total
	
	
	
	
	

	
	Bill carried to next summary
	
	
	
	

	f. RAIN WATER GOODS 

	1.
	PVC Gutter
	m
	32
	
	

	2.
	Gutter brackets@ 0.7m centres
	No
	26
	
	

	3.
	Stop ends
	No
	4
	
	

	4.
	Extra over for outlets
	No
	2
	
	

	5.
	Extra over for offsets
	No
	2
	
	

	6.
	Ditto bends
	No
	2
	
	

	7.
	PVC down pipe
	m
	3
	
	

	8.
	Excavate for tank stand n.e 1m deep
	m3
	1
	
	

	9.
	Concrete footing
	m3
	0.5
	
	

	10.
	Foundation brickwork
	m2
	6
	
	

	11.
	Plaster tank stand
	m2
	9
	
	

	12.
	Backfill to tank stand(hardcore)
	m3
	3
	
	

	13.
	Supply and fit tank 5000 litres with adaptor, tap, lid and padlock
	No.
	1
	
	

	14.
	Keep excavations free of water
	Item
	1
	
	

	TOTAL
	
	
	
	
	

	
	Bill carried to next summary
	
	
	
	

	g. FENCING
1.

Excavation of holes n.e 1m deep 

M3

68.3

Installation of 100 diameter tar poles at 2m centre to centre n.e 3m high

No

150

Backfilling and compacting using concrete 

M3

16.4

4.

Installation of 4 rows barbed wire 

m

1200

5.

Installation of galvanized 50mmx100mmx1.8m fencing

m

300

6.

Installation of 5mx1.8m galvanized gate

No

1

7.
Installation of 1.220x1.8m galvanized pedestrian gate
No
1
TOTAL
Bill carried to next summary
h. BILL OF QUANTITIES FOR CONSTRUCTION OF ABLUTION FACILITY 

FOUNDATIONS

	

	

	1.
	Excavation of Pit latrine /Pit toilet
	M3
	140
	
	

	2.
	Blockwork for pit
	M2
	44
	
	

	3.
	 Y12 steel@300x300 centres
	Tones
	1.83
	
	

	4. 
	Formwork 
	M2
	11
	
	

	5.
	Leave hole for pedestal seat
	No
	4
	
	

	6.
	Keep excavation free of water
	Item
	1
	
	

	7.
	Plaster inside walls
	M2
	49
	
	

	8.
	Backfilling and making good 
	M3
	4
	
	

	TOTAL
	
	
	
	
	

	
	Bill carried to next summary
	
	
	
	

	BRICKWORK

	1.
	Blockwork
	M2
	62
	
	

	2.
	Brickforce
	m
	131
	
	

	3.
	Meranti door frames
	No
	4
	
	

	4.
	Plaster on walls
	M2
	71
	
	

	5.
	Purlins
	m
	14
	
	

	6.
	Roof ties 4m long
	No
	10
	
	

	7.
	Corrugated roof sheets 0.5mm thick
	M2
	20
	
	

	8.
	Concrete surface bed
	M3
	4
	
	

	9.
	Topping 19mm thick smooth finish
	M2
	15
	
	

	10.
	2 coats undercoat to walls
	M2
	71
	
	

	11.
	2 coats high quality matt finishing coat to walls
	M2
	71
	
	

	12.
	Meranti doors
	No
	4
	
	

	13.
	Three lever reversable locksets (SABS Approved)
	No
	4
	
	

	14.
	Brass hinges
	Pairs
	4
	
	

	15.
	Pedestal seats with cover(3 to accommodate pre scholars and 1 for adults)
	No
	4
	
	

	16.
	110 mm diameter PVC bend
	No
	2
	
	

	17.
	6mx110mm diameter PVC pipe
	No
	2
	
	

	18.
	Catch fly for 110mm PVC pipe
	No
	2
	
	

	19.
	Backfilling to trenches
	M3
	36
	
	

	20.
	Paint to doors with high gloss enamel paint
	M2
	24
	
	

	21.
	Chalk board 3.5x1.2m with chalk ray
	No
	1
	
	

	22.
	Casting of concrete apron around the ablution facilities
	m3
	17
	
	

	
	CONTINGENCIES @10%
	
	
	
	

	
	SUB TOTAL
	
	
	
	

	
	VAT @15%
	
	
	
	

	
	GRAND TOTAL
	
	
	
	


BID NO: NLM/DP/CFMPS/2026/2027
Part C2: Pricing Data
Page 101

